PhysX Vehicle Parameters in NVIDIA Omniverse

Reference for supervisor review  —  March 2026

Purpose: This document inventories the vehicle dynamics parameters available through the PhysX Vehicle extension in Omniverse. The goal is to review this list with the supervisor and decide which parameters are essential for cockpit analysis simulation fidelity vs. which can use defaults.

⚠ Parameters are based on the PhysX 5 Vehicle SDK as exposed in Omniverse Kit. Exact property names and availability should be confirmed against the Omniverse version used in the project (Kit 106/107).

1. Drive Type Selection

Before configuring parameters, you choose a drive model. Omniverse PhysX Vehicle offers two:

| Drive Type | Description |
| --- | --- |
| Drive Basic | Simplified model. Direct torque to wheels without engine/gearbox simulation. Good for trajectory playback where realistic drivetrain behavior is not needed. |
| Drive Standard | Full drivetrain simulation: engine torque curve, gearbox, clutch, differential. Required if realistic acceleration, gear shifts, and engine braking matter for the analysis. |

⚠ Decision needed: Does cockpit analysis require realistic engine/gear behavior, or is smooth motion along trajectories sufficient? This determines whether we use Basic or Standard.

2. Complete Parameter Inventory

| Parameter | Description | Unit / Type |
| --- | --- | --- |
| Chassis / Rigid Body |  |  |
| Mass | Total vehicle mass. Affects acceleration, braking distance, suspension compression. | kg |
| Center of mass offset | Position offset from geometric center. Shifts weight distribution front/rear and left/right. | m (x, y, z) |
| Moments of inertia | Rotational inertia around each axis. Affects how the car resists roll, pitch, and yaw. | kg·m² (x, y, z) |
| Wheels (per wheel) |  |  |
| Radius | Wheel radius. Must match visual mesh. Affects ground contact point and speedometer accuracy. | m |
| Width | Wheel width. Affects tire contact patch calculation. | m |
| Mass | Unsprung mass of the wheel. Affects suspension response and ride quality. | kg |
| Max steer angle | Maximum angle the wheel can turn. Typically set on front wheels only. | degrees |
| Toe angle | Static angle of the wheel relative to the vehicle centerline. Affects straight-line stability. | degrees |
| Max brake torque | Maximum braking force applied to this wheel. | N·m |
| Max handbrake torque | Handbrake force. Typically rear wheels only. | N·m |
| Damping rate | Rotational damping of the wheel. Simulates bearing friction. | kg·m²/s |
| Suspension (per wheel) |  |  |
| Spring stiffness | How stiff the spring is. Higher = less body roll but harsher ride. | N/m |
| Spring damping | Damping rate of the shock absorber. Controls how quickly oscillations decay. | N·s/m |
| Max compression | Maximum upward travel of the suspension from rest. | m |
| Max droop | Maximum downward travel of the suspension from rest. | m |
| Sprung mass | Mass supported by this suspension unit. Usually total mass divided by number of wheels, adjusted for weight distribution. | kg |
| Camber at rest | Wheel tilt angle at rest position. Affects tire contact and grip. | degrees |
| Camber at max compression | Wheel tilt when suspension is fully compressed. | degrees |
| Camber at max droop | Wheel tilt when suspension is fully extended. | degrees |
| Suspension travel direction | Axis along which the suspension moves. Usually vertical. | vector |
| Suspension frame position | Attachment point of suspension on the chassis. | m (x, y, z) |
| Tires (per wheel) |  |  |
| Lateral stiffness | Resistance to sideways slip. Affects cornering grip. | per unit gravity |
| Longitudinal stiffness | Resistance to forward/backward slip. Affects traction and braking. | per unit gravity |
| Camber stiffness | How much camber angle contributes to lateral force. | per unit gravity |
| Friction vs slip curve | Graph defining how friction changes with tire slip. Core of the tire model. Separate curves for lateral and longitudinal. | curve |
| Tire friction table | Reference to a friction table defining grip on different surface types (asphalt, gravel, wet). | reference |
| Engine (Drive Standard only) |  |  |
| Peak torque | Maximum torque output of the engine. | N·m |
| Torque curve | Torque as a function of RPM. Defines engine power band. | curve (RPM → torque) |
| Max RPM | Engine redline. | RPM |
| Idle RPM | Engine idle speed. | RPM |
| MOI (engine) | Moment of inertia of the engine flywheel. Affects how quickly RPM changes. | kg·m² |
| Damping at full throttle | Resistance to RPM change when throttle is fully open. | rate |
| Damping at zero throttle | Engine braking effect when throttle is released. | rate |
| Gearbox (Drive Standard only) |  |  |
| Gear ratios | Array of ratios for each gear. Higher ratio = more torque, less speed. | ratio array |
| Final ratio | Final drive ratio (differential). Multiplied with gear ratio. | ratio |
| Switch time | Time to change gears. | seconds |
| Auto-box | Whether the gearbox shifts automatically. | boolean |
| Clutch (Drive Standard only) |  |  |
| Clutch strength | Maximum torque the clutch can transmit. Affects launch and gear change smoothness. | N·m |
| Differential (Drive Standard only) |  |  |
| Type | Open, limited slip, or locked. Determines how torque is split between wheels. | enum |
| Front/rear split | Torque distribution between front and rear axles (AWD only). | ratio (0–1) |
| Steering |  |  |
| Ackermann correction | Adjusts inner/outer wheel steering angles for correct turning geometry. 0 = parallel, 1 = full Ackermann. | ratio (0–1) |
| Anti-Roll Bar (optional) |  |  |
| Stiffness | Resistance to body roll in cornering. Connects left/right wheels on an axle. | N/m |
| Wheel references | Which pair of wheels this anti-roll bar connects. | prim refs |
| Aerodynamics (optional) |  |  |
| Drag coefficient | Air resistance. Affects top speed and deceleration when coasting. | dimensionless |
| Downforce coefficient | Aerodynamic downforce. Increases grip at high speed. | dimensionless |
| Reference area | Frontal area used for drag/downforce calculation. | m² |

3. Relevance to Cockpit Analysis

Not all parameters matter equally for cockpit analysis. Here is a proposed prioritization to discuss:

High priority (affects what the driver/observer sees and feels)

- Chassis mass and center of mass: determines body roll, pitch under braking/acceleration — directly visible from cockpit

- Suspension stiffness and damping: affects how the cabin moves, which impacts dashboard readability and mirror stability

- Steering max angle and Ackermann: affects turning behavior the driver directly controls and observes

- Wheel radius and width: must match visual model for correct ground contact

Medium priority (affects realism but less visible from cockpit)

- Tire friction curves: affects cornering grip and slide behavior, noticeable in Actor Mode

- Brake torque: affects stopping distance and pitch under braking

- Engine torque curve (if Drive Standard): affects acceleration feel in Actor Mode

- Anti-roll bar stiffness: affects body roll in cornering

Lower priority (fine-tuning, can default)

- Camber angles, toe angle: subtle tire behavior, unlikely to affect cockpit evaluation

- Clutch strength, gear switch time: drivetrain refinement

- Aerodynamics: only relevant at high speed scenarios

- Engine damping rates, moments of inertia: engine smoothness details

⚠ This prioritization is a starting proposal. The supervisor should confirm based on the specific cockpit analysis test scenarios the client needs.

4. Open Questions for Supervisor

1. Drive Basic or Drive Standard?

If the cockpit team primarily uses Viewer Mode (trajectory playback), Drive Basic may be sufficient. If Actor Mode needs realistic throttle/gear response, Drive Standard is needed.

2. Do we have real vehicle data for these parameters?

If the client has spec sheets for the car being tested (mass, suspension rates, gear ratios), we can input real values. Otherwise we use reasonable defaults and tune by feel.

3. Which test scenarios define what matters?

Highway cruising prioritizes different parameters than tight parking maneuvers. The scenario list drives which parameters need to be accurate vs. defaulted.

4. Surface types needed?

If all scenarios are on dry asphalt, tire friction tables can be simple. If wet or mixed surfaces are needed, the tire friction table setup becomes more involved.