# Capstone Thesis — Project Brief

RL-Learned Surface-Adaptive Emergency Evasive Maneuvers

in NVIDIA Omniverse with PhysX Vehicle Dynamics

Program: Master of Science in Autonomous Systems  |  Duration: 5–6 months  |  Credits: 60 ECTS

## Problem

When a collision is imminent, an autonomous vehicle must execute an evasive maneuver — hard braking, swerving, or both. The optimal strategy depends critically on road surface conditions: aggressive swerving works on dry asphalt but causes spin-out on ice; heavy braking on gravel produces unpredictable yaw. Today’s collision avoidance systems rely on fixed, hand-tuned reference models (e.g., Waymo’s NIEON uses a constant jerk of −30 m/s³ and maximum deceleration of −8.3 m/s² regardless of surface). These are conservative heuristics that do not adapt to road conditions and cannot exploit the vehicle’s full dynamic envelope.

## Proposed Work

This thesis trains a reinforcement learning agent to perform emergency evasive maneuvers in NVIDIA Omniverse, using PhysX for high-fidelity vehicle dynamics. The agent receives vehicle state (speed, yaw rate, slip angle) and surface friction as observations, and outputs steering, throttle, and brake commands. It must learn surface-adaptive strategies: when to brake, when to swerve, and how aggressively — depending on available grip. The agent is trained via PPO (Proximal Policy Optimization) using Stable Baselines3, with the PhysX vehicle exposed as a Gymnasium-compatible environment through Isaac Lab.

## Scenario Design

| Ego Vehicle | PhysX-simulated vehicle traveling at 50–100 km/h with full tire, suspension, and weight transfer modeling |
| --- | --- |
| Obstacle | Static or slow-moving obstacle appearing ahead at varying distances and lateral offsets |
| Surface Conditions | Domain-randomized friction: dry asphalt (μ ≈ 0.9), wet road (μ ≈ 0.5), ice (μ ≈ 0.2), gravel (μ ≈ 0.4), and mid-maneuver transitions (e.g., dry → ice patch) |
| Action Space | Continuous: steering angle, throttle, brake |
| Observation Space | Vehicle speed, yaw rate, lateral acceleration, slip angle, distance/angle to obstacle, surface friction coefficient |
| Reward Function | Collision avoidance (primary), collision severity reduction if unavoidable, vehicle stability (penalize spin/rollover), passenger comfort (jerk bounds per ISO 22839) |

## Research Questions

RQ1: Can an RL agent learn surface-adaptive evasive maneuvers that outperform a fixed-strategy baseline?

RQ2: Does the agent learn qualitatively different strategies per surface (brake-dominant on ice vs. swerve-dominant on dry)?

RQ3: How does the agent handle mid-maneuver surface transitions (e.g., dry asphalt into an ice patch)?

RQ4: Can USD-based 3D episode recording enable forensic failure analysis that flat logs cannot?

## Evaluation

| Metric | Description |
| --- | --- |
| Collision avoidance rate | % of scenarios where the agent avoids the obstacle entirely, per surface type |
| Collision severity (if unavoidable) | Impact speed and angle, mapped to MAIS injury scale (per Scanlon et al., 2023) |
| Vehicle stability | Maximum yaw rate, sideslip angle, rollover indicator during maneuver |
| Passenger comfort | Peak longitudinal and lateral jerk, evaluated against ISO 22839 thresholds |
| Strategy adaptation | Qualitative + quantitative analysis: does steering/braking ratio shift with friction? |

## Baseline Comparison

Fixed braking baseline: Maximum deceleration, no steering input (AEB-style). Derived from Waymo’s NIEON reference model parameters.

Fixed swerve + brake baseline: Hand-tuned combined maneuver using Waymo’s published lateral jerk (16 m/s³) and curvature limits (0.2 rad/m). Same parameters on all surfaces.

Both baselines are surface-agnostic, which is the core limitation the RL agent aims to overcome.

## Why Each Tool Is Necessary

| PhysX | Emergency maneuvers push the vehicle to its dynamic limits — tire saturation, weight transfer during combined braking + steering, traction loss on low-friction surfaces. Simpler models (CARLA’s bicycle model) would let the agent swerve at full speed on ice without consequences, producing unrealistic and dangerous policies. |
| --- | --- |
| RL (PPO) | The agent must learn which strategy to use based on surface conditions. This is a real-time decision-making problem with continuous actions that cannot be solved by a single hand-tuned controller. |
| USD Recording | When maneuvers fail (spin-out, rollover, obstacle clip), the failure mode is 3D and physics-dependent. USD enables synchronized scrubbing of vehicle pose, slip angles, tire forces, and metrics on a shared timeline — forensic analysis that TensorBoard plots and 2D video cannot provide. |
| Omniverse | Native integration of PhysX vehicle dynamics, Isaac Lab for Gymnasium-compatible RL environments, and USD for scene description and episode recording — all in one platform. |

## Timeline (5–6 Months)

| Month 1 | PhysX vehicle environment setup in Omniverse + Gymnasium wrapper (overlaps with internship). RL fundamentals study (PPO, reward shaping). |
| --- | --- |
| Month 2 | Scenario authoring (obstacle configs, surface conditions). Train first agent on single surface (dry). Implement baselines. |
| Month 3 | Domain randomization across surfaces. Train surface-adaptive agent. Iterate on reward function. |
| Month 4 | Evaluation suite: run trained policy + baselines across all surface/scenario combinations. USD episode recording for evaluation runs. |
| Month 5 | Failure analysis using USD recordings. Comparative results. Begin thesis writing. |
| Month 6 | Complete thesis (70–90 pages). Defense preparation. |

## Job-Market Skills Demonstrated

NVIDIA Omniverse & Isaac Lab  |  PhysX Vehicle Dynamics  |  Reinforcement Learning (PPO / SB3)  |  OpenUSD & RLxUSD  |  Python / Gymnasium  |  Vehicle Safety & ISO Standards  |  Simulation-Based AV Validation